BMU Results Report

Raya Khasati

Overview

In this project, I carried out the verification of  (BMU) which is sub unit related to RISC-V architecture using SystemVerilog and UVM methodology. The primary objective was to ensure functional correctness, reliability, and robustness of the BMU design under different operating conditions and edge cases . A complete verification environment was developed, including stimulus generation, coverage collection, and a scoreboard for result checking. During the process, I faced several challenges such as handling corner-case scenarios, debugging mismatches between the DUT and reference model, and resolving issues in constrained-random stimulus generation. By systematically analyzing waveforms, refining the testbench components, and improving the coverage model, I was able to overcome these issues and achieve functional coverage closure . This project not only ensured a high-quality BMU design but also strengthened my problem-solving skills and practical experience in verification.

Procedure & Results

I made a test class for each operation which contains all the sequences related to it , and run them sequentially ( which is actually once at time = 0 ) one by one , and each sequence contains all related test cases to this sequence type , so the full hierarchy will be something like this :

CPOP

As shown down , 6 test cases were passed out of 45 !

Here is the results which clearly explain which cases has been passed and which failed : cpop_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The mismatched results are the resultant that the DUT only counts the set bits in the least 2 Bytes !! And this is simply has been detected by the cases I suggest and by the improvements I perform for the coverage model for cpop as it will be explained later .

- The case where i provide such as conflict data path , were not handled properlly in the DUT .

Coverage Model

Note : if it is not enough clear you can visit it : cpop_fc.png

Based to this model for cpop coverage , I have write all the bins and crosses which i wrote .

Note : here it is simple design . So I know what is the cases i need to implement thats why the coverage is 100% , but in complex designs you will have hundreds or even more of test cases so you may forget some of them while you are writing the coverage model for specific operation !! .

Here is the coverage analysis I got initially :

So that means that the DUT only counts until 16 bit ( based to my cases i provide ) so ensure that ,and since the result_ff is limited which is ranged from 0-32 so we can collect its coverage easily , not as usual other outputs which they are unlimited as in addition , ..... ( except of we are concern about specific outputs ... ) so here I have implemented all the cases ( dIrect cases ) are required to produce all resulut_ff possibilities so at my side I guarantee that result_ff coverage = 100% , but as shown downward , the result_ff coverage is still 48.8% that's means the DUT didn't cover all cases , so this is led that the bug resource from DUT .

I have implemented all possible cases for result_ff as follow :

I use $countones() task , but this will not guarantee that the ones to be consecutive that's why in some cases I ran it , some cases ( which are must to be covered == cases that cpop() <=16 ) are failed , so then I noticed that even for example I have 4 ones but they may be allocated at the higher 2 Bytes so that's why the result_ff coverage may be < 48.8% so to get deeply in which cases are being passed then I defined such a function which guarantee that the 1s are consecutive as follow :

GREV

As shown down , 9 test cases were passed out of 21 !

Here is the results which clearly explain which cases has been passed and which failed : grev_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The mismatched results are the resultant that the DUT reverse the input 4 by 4 bytes for ex. 1245_224f ====> 224f_1245

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : grev_fc.png

Based to this model for grev coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

Invalid b_in test cases = 1 case , but as shown above it has been hit up to 3 times , this is due  its specific sequence ( invalid_b)  and in conflict data path & conflict reading sequences I dont care about the inputs a and b so they are being randomized that's why the hit times for the invalid bin is 3 times.

Note : the total scores for b’s bins = 20 not 21 (total number of test cases ) this is simply due to reset sequence which I don't make randomization nor initialization for the rest packet content that's why it is not covered when collect the a, b. .... signals coverage

CTZ

As shown down , 36 test cases were passed out of 88!

Here is the results which clearly explain which cases has been passed and which failed : ctz_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The mismatched results are the resultant that the DUT didn't have specific pattern for counting , so I came up with this multiple direct test cases ( as mentioned in above link ) as follow :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : ctz_fc.png

Based to this model for ctz coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

As I mentioned previouslly , here the result_ff value is limited so it is ranged from 0-32 so I can easilly collect its coverage from DUT , thats what i performed , so i made such as a direct cases to generate all the result_ff possibilities to verify if the DUT has covered them or no as follow :

Since ctz concern about consecutive zeroes that's why i implemented the function in this way ... .

So even of that , the coverage is still not full for result_ff so this indicate that the DUT logic for ctz improper .

MAX

As shown down , 2 test cases were passed out of 12!

Here is the results which clearly explain which cases has been passed and which failed : max_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The mismatched results are the resultant that the DUT MAX logic was MIN logic so the opposite , this was clear in the cases which failed as shown :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : max_fc.png

Based to this model for max coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So to ensure that the DUT is behave properly or not , I made the following :

Based to that , DUT is MIN logic for MAX , that's why the coverage is down due to this issue for result_ff coverpoint i have added .

PACK

As shown down , 4 test cases were passed out of 27!

Here is the results which clearly explain which cases has been passed and which failed : pack_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The mismatched results are the resultant that the DUT make as follow :

Result must be={least 2B_b, least 2B_a} (ba)

DUT result = {least 2B_a, least 2B_b} (ab)

So it reverse the packing order, this was clear in the cases which failed as shown:

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : pack_fc.png

Based to this model for pack coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got initially :

So to ensure that the DUT is behave properly or not , I made the following :

Based to that , DUT is reverse the order of packing , that's why the coverage is down due to this issue for result_ff coverpoint i have added .

SIEXT_B

As shown down , 82 test cases were passed out of 83 !

Here is the results which clearly explain which cases has been passed and which failed : siext_b_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : siext_b_fc.png

Based to this model for siext_b coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So to ensure that the DUT is behave properly or not , I made the following :

So based on these data , siext_b is almost implemented properly in the DUT .

SLT ( signed & unsigned )

As shown down , 50 test cases were passed out of 52 !

Here is the results which clearly explain which cases has been passed and which failed : slt_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output . ( for both signed and unsigned )

Coverage Model

Note : if it is not enough clear you can visit it : slt_fc.png

Based to this model for slt coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So to ensure that the DUT is behave properly or not , I made the following :

So based on these data , slt is almost implemented properly in the DUT .

BINV

As shown down , 36 test cases were passed out of 37 !

Here is the results which clearly explain which cases has been passed and which failed : binv_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : binv_fc.png

Based to this model for binv coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So to ensure that the DUT is behave properly or not , I made the following :

But even i have cases to cover the above bins but still it is still not hit ....

Here is the sequences :

So by logging binv is almost has been implemented properly , but the issue is how solve the issue coverage bins as explained above ...

ROR

As shown down , 34 test cases were passed out of 35 !

Here is the results which clearly explain which cases has been passed and which failed : ror_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : ror_fc.png

Based to this model for ror coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

Note : a_in & result_ff are low in coverage since the random cases are few so not all values are being hit ( or simply generated by randomize()) , so simply we can solve it by increasing the randomization cases for ex up to 1000 case …

So as shown down since

i depend on the default bins , so i have up to 64 , and the total values ( -2^31---2^31-1) are being distributed over ranges so each bin is about a range of individual values ...

So based on these data , ror is almost implemented properly in the DUT .

SRL

As shown down , 36 test cases were passed out of 37 !

Here is the results which clearly explain which cases has been passed and which failed : srl_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : srl_fc.png

Based to this model for srl coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So as shown above the coverage = 93.75% , this due in this operation i have added a cross point coverage to check directly if the DUT handle the conflict_data _path case so this what i got :

Which means that the DUT didn't detect the conflict data path cases ( they weren't handled properly ) so that's why the bin of data_path_error wasn't being hit anytime.

So based on these data , srl is almost implemented properly in the DUT .

SRA

As shown down , 37 test cases were passed out of 38 !

Here is the results which clearly explain which cases has been passed and which failed : sra_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

Coverage Model

Note : if it is not enough clear you can visit it : sra_fc.png

Based to this model for sra coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

I have covered all possible shift amount value represented by b_in as follow :

So based on these data , sra is almost implemented properly in the DUT .

SH2ADD

As shown down , 26 test cases were passed out of 33 !

Here is the results which clearly explain which cases has been passed and which failed : sh2add_test_results

After I check out my complete environment and my reference model implementation , the issues was found in the DUT :

- The case where I provide such as conflict data path , were not handled properly in the DUT so always I have random output .

- The DUT used mechanism for error detection for example in cases of overflow or even underflow test cases wasnt proper as follow :

So the result_ff value is calculated properlly but the issue was in error flag detection methdology .

Coverage Model

Note : if it is not enough clear you can visit it : sh2add_fc.png

Based to this model for sh2add coverage , I have write all the bins and crosses which i wrote .

Here is the coverage I got :

So as shown cross_a_error :

So here directly i checked the cases I have already covered , where i will have an overflow/underflow then error =1 ( assumed ) , but after data has been collected as follow :

So non bin has been hit , that means DUT didn't detect such as these errors that's why .

CSR_RDDATA_IN

It is simple operation , so by default in each operation i perform such a test case which cause a conflict reading , so here it will be usual case as follow :

Key Challenges

Initially after i done the testbench completely , and start launching it with trivial stimulus ( just as a trial ) , so i test it first with xor operation , so when i make a random test cases up to 50 or even greater , none of them is passed , then I check my reference model but everything is okey , i perform manual depugging , by logging , waveform investegation ... but not resolved !!! So at this point we can say that DUT may have the issue behind that .

Still the problem not solved 🙃. Here is I decided to check out the DUT RTL since I'm obliged to do that since none operation was working properly !!! So i mean the issue may be related to logic implementation but not in the operation itself but how the result was been multiplexed and directed into the result_ff , so that what i checked in the DUT , i have came up with this :

As shown above land operation by mistake is written with | bitwise oring instead of & , and since also the {} are being missed so when we calculate it :

(Ap.land & ap.zbb )& a | b ==>

(0 & 0 ) & a| b == 0&a|b   so here the priority from left to right so == 0|b = b

So the final result of that expression = b

And the result for xor ( correct one) must be = value

So the final answer for lout=value|b not value so that's why !!!!

So i have modified it by simply replace | by & and there is no need to put brackets since all operations are & so the same priority we have

So I think this issue will related to the rest of other operations i mean in the complete inertial design of the DUT simply due to the multiplexing to have the final answer , so after this i run my testbench and every thing is Okey !!

Second issue ( Missynchronization )

Since the UVM methodology mainly it relies on high programming mechanisms ( classes , objs .... ) so practically doesn't make sense ( i mean it is non-synthesizable code ) so we can implement and code techniques ... so based to that the testbench ideally will produce the result directly so without any delays ( generally in case we don't have delays .. )  .

But in the DUT code the concept is completely different since the code is synthesizable so the code implementation is not the same as in testing environment so here the output will not necessary to be generated once the input is provided , simply it may take multiple cycles to be generated , such these cases occurred in complicated operations as arithmetic and logical bitwise operations ( and , xor ,add , multiply , div , .... ) .

So in some operations as in xor , i have the missynchronization issue as i explained above with 3 cycles,  so the solution i performed is simple as follow :

At the  UVM :

The sequencer will pass the test case at time=0 to the driver , the driver will send the test case at posedge to the DUT , the monitor will capture the resulted from the DUT at negedge so then send them to scoreboard .... so at the end the complete process consume 1 cycle , actually the DUT at most ( since not all operations) need 3 cycles , so for these operations when the monitor capture the results actually they are not the correct ones ....

So i will send the test case 3 times , then i made such as a stack in the scoreboard , when its size = 3 so the final received packet via the monitor it is the proper result so based to it i will calculate the result based on my reference model then compare it with the DUT ones .....

Future Work

- Improve the coverage model .

- Add regression test .

- Add randomized test cases for each operation to cover as much as we can for all inputs possibilities .

- Verify the rest of BMU operations .

- Go deeply into advanced concepts related to Verification field , and apply them in this project .

Acknowledgment

I would like to sincerely thank Orion company for giving me the opportunity to complete my training. The experience has been invaluable, and I am grateful for the support and guidance from my supervisors and my peers. This training has helped me grow professionally and personally, and I truly appreciate everyone who made this journey possible !