Final Project

BMU Verification Plan

# Context

- List Of Abbreviations

- Introduction

- Design Overview

- Verification Strategy

- Testbench structure

- Verification Extra Details

# List Of Abbreviations

| Abbreviation | Definition |
| --- | --- |
| BMU | Bit Manipulation Unit |
| UVM | Universal Verification Methodology |
| DUT | Design Under Test |
| RTL | Register Transfer Level |
| OP | Operation |
| RISC-V | Reduced Instruction Set Computer Version 5 |
| Zbb | Basic Bit Manipulation Subset |
| Zba | Address Calculation Bit Manipulation Subset |
| Zbs | Single Bit Manipulation Subset |
| Zbp | Permutation Bit Manipulation Subset |
| SRL | Shift Right Logical |
| SRA | Shift Right Arithmetic |
| MSB | Most Significant Bit |
| CSR | Control Status Register |

# Introduction

Goals (Purpose) of this document

This document outlines a verification plan for BMU designed to be part of a larger processor architecture. It aims to ensure that the BMU operates correctly in all scenarios, including standard operation, edge cases, and error handling.

Scope of verification

The scope of verification for the BMU module will be limited to top-level verification, treating the BMU as a black box so we will not have access for reviewing it. We will focus on validating its inputs and outputs via its interface, such as Result and Error without considering internal details. This approach ensures the BMU’s functionality aligns with design specifications through external stimulus and response checks.

Assumptions and constraints

Constraints:

• No Constrains.

Assumptions:

- The design and verification environment uses UVM .

- Functional coverage goals are achievable and measurable.

- All external dependencies, such as clocks and resets, are provided correctly.

- The RTL design is synthesizable.

# Design Overview

The BMU is a synthesizable RTL block that implements bit manipulation functionality compliant with the RISC-V Bit Manipulation extension. The unit supports instructions from Zbb, Zbs, Zbp, and Zba subsets and handles bit-level arithmetic, logical, and data packing operations. It is integrated into a larger processor pipeline and contributes to the integer execution path.

Key features

- Complement

- with RISC-V Bit manipulation extensions as follow:

Zbb : CLZ , CTZ , CPOP , MIN , MAX , SEXT.B , SEXT.H , ANDN , ORN , XNOR .

Zbs : BSET , BCLR , BINV , BEXT .

Zbp : ROL , ROR , PACK , PACKU , PACKH , GREV ( subset ) , GORC ( subset ) .

Zba : SH1ADD , SH2ADD , SH3ADD .

Each extension as shown above is assumed to be responsible to perform the assigned operations for each one.

- Intergrated control and data path: each operation will have specific control data path to activate it properly.

- Flushing and prediction support for branch misprediction recovery scenarios: in case the specified path is not supported or simply not found it will be handled properly .

- Configurable instruction enablement using compile-time parameters .

- Robust error detection logic: The BMU includes mechanisms to detect and respond to invalid operations or unexpected conditions or even wrong outputs.

- Clock-gated PC register for low-power optimization.

Block diagram and interface

The interface and signals details are listed below in the table:

Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 bit | input | Clock signal to the module |
| rst_l | 1 bit | input | Active-Low synchronous reset , reset all internal registers of the design to its initial state |
| a_in | 32 bits | input | Represents the first operand of BMU module |
| b_in | 32 bits | input | Represents the second operand of BMU module |
| scan_mode | 1 bit | input | Scan test mode control |
| valid_in | 1 bit | input | Instruction valid flag , indicate if the input instruction is valid or not |
| ap | Struct | input | Decoded instruction control signals (supported instructions by the BMU) |
| csr_ren_in | 1 bit | input | CSR read-enable |
| csr_rddata_in | 32 bits | input | CSR read data |
| result_ff | 32 bits | output | Final computed result |
| error | 1 bit | output | Error Indicator |

# Verification Strategy

## Methodology

UVM will be used to create a scalable and reusable verification environment. The reference model will be written in SystemVerilog HDL as a task included in the scoreboard, the BMU testbench will include driver , monitor, scoreboard, subscriber for functional coverage (UVM components ) .

## Selection of Verification Tests

Tests will cover all aspects of the BMU functionalities, including:

- logical  Operations

This section defines logical operations performed by the BMU, primarily controlled by the ap.op field.The operation mode depends on the ap.Zbb extension bit to be asserted and requires all other control fields to be disserted.

NOTE : Zbb extension support the negations of logical ops,for ex. in Oring Zbb=0; so the Zbb will decide if it is usual or negation logical operation .

  - Standard Bitwise OR Operation

- Basic ORing between any 2 inputs. ( avg case )

- ORing between both 0-bits  input values (32’b0).

- ORing between both 1-bits  input values (32’b1).

- ORing between zero and non-zero inputs (0 ^ ...).

- ORing between one and non-one inputs (1 ^ ...).

- ORing between alternating input values (inverted 2nd input) (ex. 10 ^ 01).

  - Standard Bitwise Invertor OR Operation

- Basic NORing between any 2 inputs.

- NORing between both 0-bits input values (32’b0).

- NORing between both 1-bits input values (32’b1).

- NORing between zero and non-zero inputs (0 ~| ...) .

- NORing between one and non-one inputs (1 ~| ...).

- NORing between alternating input values (inverted 2nd input) (ex. 10 ^ 01).

  - Standard Bitwise XOR Operation

- Basic XORing between any 2 inputs  .

- XORing between both 0-bits input values( 32’b0 ) .

- XORing between both 1-bits input values ( 32’b1 ).

- XORing between zero and non-zero inputs (0 ^ ...).

- XORing between one and non-one inputs (1 ^ ...).

- XORing between alternating input values (inverted 2nd input )  (ex. 10 ^ 01 ) .

  - Standard Bitwise Invertor XOR Operation

- Basic NXORing between any 2 inputs  .

- NXORing between both 0-bits input values (32’b0).

- NXORing between both 1-bits input values (32’b1) .

- NXORing between zero and non-zero inputs ( 0 ~^ ....).

- NXORing between one and non-one inputs ( 1 ~^ ....).

- NXORing between alternating input values (inverted 2nd input )  (ex. 10 ^ 01 ) .

NOTE : In bitwise logical operations there is no need to test the max and min values (represented by 32’b0000... , 32’b1111... ) combinations since they are bitwise operations, so we don't have overflow and under flow issues.

- Shifting and Masking Operation

The Shifting and Masking operations include logical shifts, arithmetic shifts, rotates, and bit-level operations. Each operation is enabled by a dedicated control signal in the ap register. Only one shifting/masking operation should be active at a time, and all other ap.* fields must be 0, unless explicitly required by the operation.

NOTE: csr_ren_in must be 0 for any valid operation.

NOTE : We will not concern about min&max values in this type of operations since there are not edge cases here and it will affect the bit itself so no matter .

  - Right Logical Shift Operation (SRL)

SRL operation is part of the BMU’s shifting functionality. It is controlled by the ap.srl signal and performs a logical (zero-fill) right shift of a_in by the amount specified in the lower 5 bits of b_in.

NOTE : since a_in&b_in are 32-bit inputs so since the shift amount will depend on the least 5-bits of b_in so the max amount = 2^5 = 32 time! So that means we will shift a_in 32 time at most which = 0 at the end iteration! So the b_in values are ranged from 0 to 31 .

NOTE : amount means no sign always positive  by default.

Click here to for displaying the test cases I performed : srl_cases

  - Right Arithmetic Shift Operation (SRA)

SRA operation is part of the BMU’s shifting functionality. It preserves the sign of a signed number during the shift by extending the sign bit as it shifts right. The shift amount is determined by the lower 5 bits of b_in.

NOTE : since a_in&b_in are 32-bit inputs so since the shift amount will depend on the least 5-bits of b_in so the max amount = 2^5 = 32 time! So that means we will shift a_in 32 time at most which = 0 at the end iteration! So the b_in values are ranged from 0 to 31 .

NOTE : amount means no sign always positive  by default.

Click here to for displaying the test cases I performed : sra_cases

  - Rotate Right Operation ( ROR )

ROR operation is a bit manipulation instruction that rotates the bits of a_in to the right by the number of positions specified in the lower 5 bits of b_in. Bits shifted out from the LSB side are reintroduced at the MSB side.

NOTE : since a&b are 32-bit inputs so since the shift amount will depend on the least 5-bits of b so the max amount = 2^5 = 32 time! So that means we will shift a_in 32 at most which = the original value  at the end iteration !

NOTE : amount means no sign always positive  by default.

Click here to for displaying the test cases I performed : ror_cases

  - Bit Inverse Operation (BINV)

BINV operation toggles (inverts) a single bit in the operand a_in at the bit position specified by the lower 5 bits of b_in. All other bits remain unchanged.

Click here to for displaying the test cases I performed : binv_cases

  - Shift Left by 2 and Add (SH2ADD)

SH2ADD operation shifts a_in left by 2 bits and adds the result to b_in. This operation is valid only when ap.zba is asserted (equal to 1).

NOTES :

- Since there is no critical cases for the bitwise shift operation itself , but we have for the addition (overflow & underflow ) ! So here we need to concern about max and min values

- 0x7fffffff + any positive value == > overflow

- 0x80000000+ any negative value == > underflow

- Neg+pos values == neither overflow nor underflow issues .

- Overflow & underflow  can be detected using the carry&borrow flags  and the MSB .

- The shift type that the op perform is logical .

Click here to for displaying the test cases I performed : sh2add_cases

- Arithmetic Operations

- Subtraction (a – b )

- Both inputs =0 , result=0 , no error

- Both inputs =32’b1  result = 0 , no error

- Equal inputs in both sign and magnitude , result =0 , no error

- Avg case (any 2 inputs )

- A=max   -   b=max    result=0,no error

- A=min    -    b=min    result=0,no error

- Equal inputs only in magnitude :

- avg case ( any 2 inputs )

- a=min     -   b=max       result=--- , error : overflow

- a=max    -    b=min         result = 1 , no error

- A=0   -   b=-2^31(max)   result=2^31 , error : overflow

- a = 2^31 - 1, b = -1   result=2^31 , error:overflow

- a = -2^31, b = 1   result:-2^31 –1 , error:overflow

- a = -2^31, b = 1   result : -2^31  - 1, error:underflow

- A=0   b=max   result=-max ,no error

- A=0   b=min    result=-min   , error:overflow

Note : result akbar mn the inputs ( + or - ) ( so largeset if are +) ( or smallest if are - ) === > this is overflow

Overflow : Happens when the result of an operation exceeds the maximum value that can be represented by the data type.

Underflow : Happens when the result of an operation goes below the minimum value that can be represented by the data type.

- Bit Manipulation Operations

- Set on Less Than (Unsigned SLT)

The SLT operation compares two operands (a_in and b_in) and sets the result to 1 if a_in is less than b_in. The comparison is signed if ap.unsign = 0 .

So to make comparison :  SUB will be introduced between a & b respectively or as I explain downward

Result=  a<b ? 1 : 0

NOTE : since it is unsigned op so all inputs are treated as +  values

- Same inputs then result=0 , no error .

- Different inputs , simply check if the 1st 1 we came up (from left) in A input but the corresponding bit in b is 0 then result =0 ( a not less than b ) , else result =1 ( logically ) .

Click here to for displaying the test cases I performed : unsigned_slt_cases

- Set on Less Than (SLT signed == default)

Click here to for displaying the test cases I performed : signed_slt_cases

- Count Trailing Zero Bits (CTZ)

The CTZ operation counts the number of consecutive zeros in a_in, starting from the least significant bit (LSB) and moving toward the most significant bit (MSB). It is useful in bit scanning and locating the first set bit from the right.

NOTE : Ctz =0-32 zero , so we can came up with all cases for the result !!

NOTE : No edge cases !! All cases are usual since it is simple counting process .

Click here to for displaying the test cases I performed : ctz_cases

- Count Set Bits / Population Count (CPOP)

The CPOP operation counts the total number of bits set to 1 in the input a_in. This is commonly used in bit manipulation algorithms and for parity or weight calculations.

NOTE : No edge cases !! All cases are usual since it is simple counting process

Click here to for displaying the test cases I performed : cpop_cases

- Sign Extend Byte (siext_b)

The siext_b operation sign-extends the lower byte (bits [7:0]) of the input a_in to the full 32-bit width by replicating the sign bit (bit 7) into the higher bits [31:8].

NOTE : No edge cases !! All cases are usual since it is simple counting process but we need to ensure how to isolate the LS byte , so we only need to concern about these bits a_in[7] & a_in[31] ( to observe out the conversion of the sign bit ) .

Click here to for displaying the test cases I performed : siext_b_cases

- Maximum (max)

The max operation compares two signed integers (a_in and b_in) and returns the larger value. It is part of the RISC‑V Bit Manipulation (Zbb) extension.

NOTE : To enable the MAX we must enable sub , this is for the RISC-V hardware    low level implementation since it does not have such as a comparator as we have !! Thats why here it is very simple to perform max , min ops since it is direct comparison !!

Click here to for displaying the test cases I performed : max_cases

- Pack (pack)

The pack operation concatenates the least significant 16 bits of b_in and a_in to form a 32‑bit result. This is part of the RISC‑V Bit Manipulation (Zbp) extension.

NOTE : result= { L2Bytes(b) , L2Bytes(a) }

NOTE : No edge cases !! All cases are usual , but we need to ensure how to isolate the L2Bytes .

Click here to for displaying the test cases I performed : pack_cases

- Byte-Reverse Register (grev)

The grev operation reverses the order of bytes in a_in. Only byte ordering is reversed, not the individual bits within each byte. This operation is controlled by ap.grev and requires b_in to be equal to 24 (5'b11000) in its lower 5 bits.

NOTE :  why b must = 24d ,  simply this is depend in RISC-V interial structure so that means perform byte-level reversal on input a_in .

NOTE : No edge cases !! All cases are usual  since it is ordering process !, but we need to ensure how to implement the reverse implementation and how to capture each byte .

Click here to for displaying the test cases I performed : grev_cases

- CSR operations

This section covers CSR-related logic in the Bit Manipulation Unit (BMU), including bypass read and write operations. These operations are influenced by the fields in the ap control structure and selected inputs (csr_ren_in, csr_rddata_in, csr_write, etc.).

NOTE : CSR it is BMU register so we can read it or write on it .

- CSR Read Data

We will read the CSR content and assign it to the result

Click here to for displaying the test cases I performed : read_csr_cases

- Unsupported Opcode Handling

We have already covered this type of testing , by providing conflict_data_path sequence for each valid operation .

- Random cases

We have already covered this type of testing , by providing random sequence for each operation  , so the inputs a_in & b_in will be randomized .

# Testbench structure

Also I will use a Subscriber which will be found inside the Environment for coverage issues

## Tests

For each operation i will implement a test which contains all the pre-mentioned sequences

## Coverage targets

- Code (line) coverage

- Functional coverage

- Toggle coverage

- Path coverage

- Cross coverage ( advanced , check multiple options once and for testing direct sequence .. )

# Verification Extra Details

Components : driver , monitor , sequencer , sequence_item , agent , environment ,scoreboard , subscriber .

Interfaces: middle-ware interface between the DUT and the top testbench module

Tools and technologies:

- MobaxTerm Server .

- Intergrated Metrics Center (IMC) .

- Cadence Verisium for debugging .

- Visual Studio Code .

Regression Tests:

- Create Regression test to test all cases i have set up and run them together instead of running them individually in the top testbench module .

Coverage Reports: will be attached later

Verification Results: will be attached later